## Code Explanation

By Yasser Elhoujjaji

Here's a detailed Mermaid flow diagram with explanations for the provided code:Explanation:

1. Global Variables:

- Defines various LED pins and their states.

- Stores the node ID and other constants.

- Keeps track of the previous time the device was rebooted.

2. Includes:

- Includes the DFRobot_SHT20 library for the temperature and humidity sensor.

3. LoRa Settings:

- Defines the LoRa physical layer parameters, such as frequency, spreading factor, bandwidth, code rate, preamble length, and transmit power.

4. Setup:

- Initializes the LED pins as outputs.

- Initializes the serial communication at 115200 baud.

- Checks and sets the node device work mode.

- Prints the system information, including hardware ID, model ID, API version, firmware version, and AT command version.

- Sets the LoRa parameters, such as frequency, spreading factor, bandwidth, code rate, preamble length, and transmit power.

- Registers the send callback function.

- Initializes and checks the SHT20 sensor.

- Powers on the sensor.

5. Main Loop:

- Reads the temperature and humidity data from the SHT20 sensor.

- Sends the sensor data to the gateway.

- Enters sleep mode for 10 minutes.

- Checks if 24 hours have passed since the last reboot and reboots the device if necessary.

6. Read Sensor Data:

- Reads the humidity and temperature values from the SHT20 sensor.

7. Send Data to Gateway:

- Turns on the send LED.

- Prints the temperature and humidity data.

- Formats the sensor data into a message string.

- Converts the message string to a byte array.

- Sends the data via LoRa P2P.

- Turns off the send LED.

8. Enter Sleep Mode:

- Turns on the sync LED.

- Prints the timestamp before entering deep sleep mode.

- Enters deep sleep mode for 10 minutes.

- Prints the timestamp after waking up.

- Turns off the sync LED.

9. Reboot:

- Checks if 24 hours have passed since the last reboot.

- Reboots the device if the interval has been exceeded.

The code sets up a temperature and humidity sensor, reads the sensor data, and sends the data to a gateway using LoRa P2P communication. It also includes a sleep mode to conserve power and a reboot mechanism to ensure the device remains operational.